Design Plan and Documentation

Project Title: Lego Segway (2 wheel balancing and RC robot)

Team Members:
	- Team Lead: John Gerken
	- Design Lead: John Gerken
	- Programming Lead: John Gerken
	- Emotional Support Lead: Bella Gerken (Dog)

Client-Provided Goal / Problem to Solve:
Design, assemble, and program a robot that can simultaneously balance on 2 wheels while being remotely controlled. This robot should be constructed out of Legos to maximize future versatility and consumer repairability. The robot should be programmed with Python to improve flexibility.

Solution to Above Challenge:
Hardware: I will use the Lego Mindstorms EV3 platform for the construction of the robot due to the fact that it provides all the necessary parts and elements needed to achieve the objective.

Design: Use 2 large wheels on the sides with a powerful motor attached directly to each. Use a 3rd, smaller, smooth, unpowered wheel on the rear to act as a tripod for the robot to sit on when turned off. The rear wheel is smooth to allow the robot to turn while sitting on the rear wheel. I will use a Gyro Sensor to detect the current angle of the robot and an Infrared sensor to receive input from the remote.

Software: I will incorporate a PID (Proportional, Integral, Derivative) controller script into my program in order to help the robot balance. This, inside a continuous feedback loop with data provided from the gyro will cause the robot to balance. If the robot starts to tilt forward, the robot will detect this and turn the wheels forward, and if it starts to tilt backward, it will turn the wheels backwards. This is in order to keep the robot/ground contact point roughly under the robots center of mass so the robot can stay balanced.

Plugs:
	- Left Motor: Port D
	- Right Motor: Port A
	- Gyro Sensor: Port S1
	- Ultrasonic Sensor: Port S4

Program Flowchart: